MAPS ASIC1 test board guide v0.1

Power supply

The board is powered from two-terminal screw posts connector J16, located at the bottom left corner of the board. The polarity of the connector is drawn in silkscreen below the connector. If power supply is connected wrongly the board would not work, but nothing would get damaged.

Requested power supply is 5V / 3A max.

Current measurement jumpers

These jumpers are placed in series with power supply lines. During normal operation, the jumpers should be fitted.

J1	VDD2V5dig

J2	VDD3V3dig

J3	VDD1V8dig

J4	VDD1V8mso

J5	VDD1V8pix

J6	VDD1V8dco

J7	VDD1V8sram

J8	VDD1V8aco

J9	VDDO

VRST setting jumper

The jumper J24 in the upper right corner of the board is used for setting the source of VRST for the sensor. Shorting pins 1 and 2 sets the source of VRST to be DAC; shorting pins 2 and 3 sets the source of VRST to be VDD1V8pix.

Substrate connection jumper

The jumper J23 below the sensor is used for substrate connection of the sensor. Placing a jumper between pads 1 and 2 connects the substrate to analog ground; placing a jumper between pads 2 and 3 connects the substrate to digital ground. Placing no jumper would leave substrate floating.

Probing headers

J25 and J26 are headers used for probing. The order of signals is as follows:

J25:	1 – DEBUG34_P1B

2 – DEBUG34_P2B

3 – DEBUG34_P3B

4 – DEBUG34_RST

5 – DEBUG12_MA0

6 – DEBUG_TC0

J26	1 – DEBUG_HITOUT1

2 – DEBUG_HITOUT2

3 – DEBUG_HIT200

4 – DEBUG_HIT600

Analog outputs

The coaxial connectors J27 – J32 are used for probing of analog outputs from the sensors:

J27	DEBUG_RSTVAL2

J28	DEBUG_SIGVAL2

J29	DEBUG_RSTVAL1

J30	DEBUG_SIGVAL1

J31	DEBUG_COMPN_OUTB

J32	DEBUG_COMPN_OUT

Logic analyzer connectors

There are three LA connectors on the board, marked in silkscreen as PORTA (J17), PORTB (J19) and PORTC (J18). Please note that PORTA and PORTC are swapped compared to preliminary configuration.

| Pin # | PORTA | PORTB | PORTC |
| --- | --- | --- | --- |
| 1 | N/A | N/A | N/A |
| 2 | N/A | N/A | N/A |
| 3 | N/A | N/A | N/A |
| 4 | N/A | N/A | N/A |
| 5 | N/A | N/A | N/A |
| 6 | N/A | N/A | N/A |
| 7 | READ_ENABLE_L[1] | CONFIG_FAST_PHI1 | CONFIG_IN |
| 8 | READ_ENABLE_L[0] | CONFIG_FAST_PHI2 | DOUT[0] |
| 9 | READ_ENABLE_R[1] | CONFIG_FAST_PHI3 | DOUT[1] |
| 10 | READ_ENABLE_R[0] | CONFIG_FAST_RSTB | DOUT[2] |
| 11 | CONFIG_OUT_TOP | CONFIG_SLOW_PHI1 | DOUT[3] |
| 12 | SENSE_Enb | CONFIG_SLOW_PHI2 | DOUT[4] |
| 13 | OPMUX[0] | CONFIG_SLOW_PHI3 | DOUT[5] |
| 14 | OPMUX[1] | OF1 | DOUT[6] |
| 15 | READBACK_OUT | OF2 | DOUT[7] |
| 16 | READBACK_PARALLEL_LOAD | OF3 | DOUT[8] |
| 17 | READBACK_RSTB | OF4 | DOUT[9] |
| 18 | READBACK_TEST_IN | ORE12[0] | DOUT[10] |
| 19 | N/A | ORE12[1] | DOUT[11] |
| 20 | N/A | ORE34[0] | DOUT[12] |
| 21 | N/A | ORE34[1] | DOUT[13] |
| 22 | N/A | READBACK_FAST_PHI1 | DOUT[14] |
| 23 | N/A | READBACK_FAST_PHI2 | DOUT[15] |
| 24 | N/A | READBACK_FAST_PHI3 | DOUT[16] |
| 25 | N/A | SAMPLE_RESETB34 | DOUT[17] |
| 26 | N/A | TIMECODE[0] | DOUT[18] |
| 27 | N/A | TIMECODE[1] | DOUT[19] |
| 28 | N/A | TIMECODE[2] | DOUT[20] |
| 29 | N/A | TIMECODE[3] | DOUT[21] |
| 30 | N/A | TIMECODE[4] | DOUT[22] |
| 31 | N/A | TIMECODE[5] | DOUT[23] |
| 32 | N/A | TIMECODE[6] | DOUT[24] |
| 33 | N/A | TIMECODE[7] | DOUT[25] |
| 34 | N/A | TIMECODE[8] | DOUT[26] |
| 35 | N/A | TIMECODE[9] | DOUT[27] |
| 36 | N/A | TIMECODE[10] | DOUT[28] |
| 37 | N/A | TIMECODE[11] | DOUT[29] |
| 38 | N/A | TIMECODE[12] | DOUT[30] |

Test points

There are four test loops on the board TP1-TP4, marked in silkscreen ANALOG GND or DIGITAL GND.

Optional components

There are six optional capacitors on the board, C179-C183. Those could be used to limit bandwidth of operational amplifiers driving analog outputs of the board (see schematics).

The component marked as R86 is actually a place holder for 0603 0ohm resistor to either VDD or GND, or neither of them (see schematics). If left unpopulated, then the outputs of the octal DAC IC10 would power up to mid range; if a 0603 0ohm resistor is placed between centre pad and the pad denoted VDD, the outputs of IC10 would power up to top range (this is presumably a wanted configuration, so the resistor need to be mounted by hand); if a 0603 0ohm resistor is placed between centre pad and the pad denoted GND, then the outputs of IC10 would power up to zero.

Close match components (tbc)

Due to problems with sourcing of components, it is possible that initial batch of three boards have got some “close match” alternatives mounted. Those are:

R74 (exact value 12k4, on first three boards 12k), defining the current debugsfbias

R76 (exact value 48k7, on first three boards 47k), defining the current ISENSE_COLREF.