Problem Report 	Report Number:  4

Project Name:  TeraPixel APS for CALICE    v1.1

Item:  Injection from DEBUG_HIT_OUT onto DEBUG_RSTVAL1

Problem

An injection of signal from the comparator to the shaper input signal in the test pixel is observed during analog test pixel investigations A dependence on VDD1V8dig is seen in the magnitude of the injected signal on DEBUG_RSTVAL1 (yellow) and no effect on DEBUG_SIGVAL1 (blue) Investigation reveals this is NOT occurring in the pixel, but rather occurs in the line from in-pixel source-follower (for the analog signal) and digital buffer (for the digital HIT signal)

Originator (Sign/Date) JC

Project Manager (Sign/Date) JC

Remedial Action

Understanding of the problem is sufficient for operation of these test pixels. No suggested action. Be aware if re-spun, but unlikely to reoccur as is if scaled to a larger format sensor.

Project Manager (Sign/Date)