Problem Report 	Report Number:  5

Project Name:  TeraPixel APS for CALICE    v1.1

Item:  Configuration Registers: Power droop in buffer supplies

Problem

- Were reporting many errors and not holding the correct data - Problem due to different power domains and high loading of “slow” clock drivers

Originator (Sign/Date) JC

Project Manager (Sign/Date)

Remedial Action

- Fixed by driving one of the power domains at 1.6v (previously 1.8v) - Implemented using the power module developed for TPAC1.0 with updated resistor values and driving VDD1V8sram through J7 - Proved, reliable fix provided 2% tolerance on resistors on power module board.

Project Manager (Sign/Date) JC