Review Approval Form

Type of Review:  IDR (pixel schematics)

Project Name:  TeraPixel APS for CALICE

Documents Reviewed

Latest top level schematics : PreShape simulation results document PreSample simulation results document Comparator simulation results document Referred to but not explicitly reviewed: Logic and top-level simulation results document Attendees: JC, RT, Paul Dauncey, Nicola Guerrini, Mark Prydderch, Mike Tyndel.

Comments

Full minutes taken by Paul Dauncey, available on web at http://www.hep.ph.ic.ac.uk/calice/maps/idr1/idr1.html PreShape pixel Investigate offset in different traces on preshape example Check current consumption of pixels for very large signals (amplifiers turning off might raise power rails) Check leakage figures for the diodes Update the documents (eg power in comparators) so they can form accurate documentation for the chip, and add page numbers Run a simulation for power supply rejection – set the spec for the chip power supply? Add expected parameter spread to the table of risks for pixel components Evaluate/prove effectiveness of using two series feedback capacitors instead of one  change schematics as req Review in-pixel resistors in same context of matching – may require two parallel resistors if small hi-poly is used. PreSample pixel Quantify component risks as for pre-shape [Note: Cin/Cfb references swapped in doc pg17] Make the feedback capacitor from two caps in series Check AC analysis of analog circuits, phase margin etc? (also for pre-shape pixel) It was queried whether the pixel circuits will be re-optomised when the input capacitance is known – if time is available then some quick checks can be made but no major changes would be anticipated. Comparator Repeat threshold-timing sim for 120mV case (only 30mV and 60mV considered) Check transient noise with Eldo & compare with spectre results Investigate discrepancy in the with-adjustment results (dialogue contradicts plot labelling) Logic There was not time to evaluate the logic parts in full details, although a top-level summary was given which yielded the following comments: Add a parallel-load serial-output shift register to read back the mask & trim settings that were programmed – very useful for validating what was written. Separate the “overflow” outputs for each pixel type; Data mux and row encoder designs to be completed. Will hold a separate full review for the logic section, to include other interested parties, ie DAQ representatives.

Approval: emails indicating approval are an adequate substitute for hardcopy signatures Review Report agreed Group Leader (sign/date) as required by the Project Management Plan Customer (sign/date) as required by the Project Management Plan Others – as stated in the Project Management Plan Review Report agreed and any changes incorporated Project Manager (sign/date) always required