Problem Report 	Report Number:  2

Project Name:  TeraPixel APS for CALICE

Item:  ASIC1 Sensor design: Sense amplifier bias

Problem

(pad) (sense amplifier bias block) ISENSEBIAS 3.3v LV HV HV HV The sense amplifier bias chain includes some high-voltage tolerant transistors (thicker oxide) as the biasing requires a 3.3v power supply. The use of a general-purpose bias pad means there is a low voltage (LV) nmos transistor mirroring current into a high voltage (HV) nmos transistor. The current mirroring is therefore incorrect as the transistors have different threshold voltages.

Originator (Sign/Date)

Project Manager (Sign/Date)

Remedial Action

External bias currentInternal current if correct LV nmos had been usedActual current with HV nmos in mirror circuit65u12.8u0.5u270u53u10.9u300u59u15.1u360u70u17.3u Simulation shows the bias current that was originally specified would have set a very low current in the sense amplifiers due to the mismatch in the bias circuits. By increasing the external current to 300uA (typical) the correct range of currents can be achieved in the sense amplifiers. This is well out of the original range of the bias DACs so resistor R84 on the sensor board must be changed to 7k5ohms giving a suitable range in the right region. DESIGN FIX: Change ISENSEBIAS reference transistor to HV type (thick oxide define layer)

Project Manager (Sign/Date)